ELEC281 Mini Challenge: Design Stage & Circuit Specification

Interactive Digital LED Hourglass with Accessibility & Dual-Sided Interface
Module: ELEC281 - Applied Electronic Skills and Engineering Practice | Box 34

# 1. Executive Summary & CDIO Alignment

This engineering design specification fulfills the Design Stage deliverable for the ELEC281 Mini Challenge (Digital Hourglass). The project transforms the traditional passive hourglass into a smart, accessible timer featuring: (1) A front visual display with sand particle physics simulated on two cascaded 8x8 LED matrices; (2) A rear 4-in-1 LED matrix displaying accurate numerical countdowns (MM:SS); (3) A 4x4 matrix keypad enabling custom user duration setting; (4) An auditory feedback loop powered by a passive buzzer, designed specifically for blind/visually-impaired users.

# 2. Engineering Consistency & Traceability Matrix

To guarantee 100% feasibility during the Implementation Stage, all hardware connections, software parameters, and architectural flows have been cross-verified against the operational source code (DigitalHourglass_Core.ino) and physical circuit schematics. Table 1 outlines this exact parity.

| System Function | Hardware Component | Arduino R4 Pin / Bus | Code Identifier in .ino | Validation Status |
| --- | --- | --- | --- | --- |
| Visual Front (Top Sand) | MAX7219 8x8 Matrix (#0) | Pin 11 (DIN), 10 (CS), 13 (CLK) | lc.setLed(0, ...), DIN_PIN=11 | Verified Match |
| Visual Front (Bottom Sand) | MAX7219 8x8 Matrix (#1) | SPI Daisy-Chain (DOUT->DIN) | lc.setLed(1, ...), NUM_MATRICES=6 | Verified Match |
| Visual Rear (Digital MM:SS) | MAX7219 4-in-1 Matrix (#2..5) | SPI Daisy-Chain (DOUT->DIN) | drawDigit(2..5, ...) | Verified Match |
| Tilt / Flip Sensing | ICM20948 9DOF IMU | I2C: SDA, SCL (3.3V Rail) | icm.begin_I2C(), icm_accel->getEvent() | Verified Match |
| Auditory Accessibility | Passive Buzzer Module | Pin 9 (PWM) & GND | BUZZER_PIN=9, tone(9, freq) | Verified Match |
| Keypad Row Pins (1..4) | 4x4 Membrane Keypad | Pins 2, 3, 4, 5 | byte rowPins[4] = {2, 3, 4, 5} | Verified Match |
| Keypad Col Pins (5..8) | 4x4 Membrane Keypad | Pins 6, 7, 8, 12 | byte colPins[4] = {6, 7, 8, 12} | Verified Match |
| Non-blocking Scheduler | MCU Hardware Timer | Internal System Millis | currentMillis - lastDropTime >= dropInterval | Verified Match |

# 3. High-Level Hardware Flow Diagram (Architecture)

The hardware architecture separates the system into distinct functional subsystems: Power Distribution, Input/Sensing, Main Processing Unit, and Multi-modal Feedback. Crucially, the 5V power bus drives the high-current display matrices, while the 3.3V low-dropout regulator powers the sensitive inertial motion sensor.

+---------------------------------------------------------------------------------------------------+
|                                    HIGH-LEVEL HARDWARE FLOW                                       |
+---------------------------------------------------------------------------------------------------+
|                                                                                                   |
|  [POWER SOURCE] ===> USB 5V ===> Arduino Uno R4 WiFi (R7FA4M1 @ 48MHz)                            |
|                                       |                  |                                        |
|                                 (3.3V Rail)          (5V Rail)                                    |
|                                       |                  |                                        |
|  [INPUT SUBSYSTEM]                    v                  v            [OUTPUT SUBSYSTEM]          |
|  +--------------------+       +---------------+   +---------------+   +------------------------+  |
|  | 4x4 Keypad         | ====> | ICM20948 IMU  |   | Display Bus   |   | Front 2x MAX7219       |  |
|  | (8x GPIO Pins 2..8)|       | (I2C: SDA/SCL)|   | (5V & GND)    |   | (Hourglass Sand Sim)   |  |
|  +--------------------+       +---------------+   +---------------+   +------------------------+  |
|            |                          |                  |                        ^               |
|            | (Key Strobe)             | (Orientation)    | (Power)                | (SPI DOUT)    |
|            v                          v                  v                        |               |
|  +----------------------------------------------------------------+       +--------------------+  |
|  |                     ARDUINO UNO R4 CENTRAL CONTROLLER          | ====> | Rear 4-in-1 Matrix |  |
|  |           (Non-blocking FSM & Particle Physics Engine)         | (SPI) | (MM:SS Countdown)  |  |
|  +----------------------------------------------------------------+       +--------------------+  |
|                                       |                                                           |
|                                (PWM Pin 9 @ tone)                                                 |
|                                       v                                                           |
|                               +-------------------+                                               |
|                               |  Passive Buzzer   | (Blind-friendly auditory cues:                |
|                               |  (Sound Feedback) |  Key clicks, 2x flip beep, alarm loop)        |
|                               +-------------------+                                               |
+---------------------------------------------------------------------------------------------------+

# 4. High-Level Software Flow Diagram (FSM)

The firmware is structured around an event-driven Finite State Machine (FSM). All timing loops operate on delta-time comparisons using millis(), completely avoiding delay() calls. This ensures that physical flips and keyboard interrupts are detected instantaneously regardless of display refresh cycles.

+---------------------------------------------------------------------------------------------------+
|                             FINITE STATE MACHINE (FSM) SOFTWARE FLOW                              |
+---------------------------------------------------------------------------------------------------+
|                                                                                                   |
|  [1. BOOT_INIT]                                                                                   |
|    - Initialize Serial (115200 bps), SPI Bus, I2C Bus, Keypad pins                               |
|    - Wake and clear 6x MAX7219 matrices, play startup chime: tone(9, 1000, 200)                   |
|    - Load default duration: totalTimeMs = 300,000ms (5 mins)                                      |
|                           |                                                                       |
|                           v                                                                       |
|  [2. IDLE_READY / SETTING MODE]                                                                   |
|    - Rear display shows configured time (default '0500')                                          |
|    - If '0'-'9' pressed: Left-shift buffer, chirp tone(9, 2000, 50), update rear display          |
|    - If 'C' pressed: Clear buffer to '0000', chirp tone(9, 1000, 100)                             |
|    - If '#' pressed: Commit duration, calculate dropInterval = totalTimeMs / 64, tone(1500, 200) |
|                           |                                                                       |
|                           | (Event: IMU Accel Z Inversion > 5.0 or < -5.0 m/s^2)                 |
|                           v                                                                       |
|  [3. TIMING_ACTIVE (RUNNING)]                                                                     |
|    - Play double confirmation beep: tone(1200Hz) x2 for accessible blind feedback                |
|    - Map active top matrix (0 or 1) and bottom matrix                                             |
|    - Every loop: non-blocking check:                                                              |
|        * Check millis() - lastDropTime >= dropInterval: topSand--, bottomSand++                   |
|        * Update rear display with remaining MM:SS                                                 |
|        * Poll IMU: if user flips 180 deg mid-way -> reorient matrices, double-beep, restart timer |
|                           |                                                                       |
|                           | (Condition: elapsed >= totalTimeMs && topSand == 0)                   |
|                           v                                                                       |
|  [4. ALARM_DONE]                                                                                  |
|    - Rear display locks to '0000'                                                                 |
|    - Trigger non-blocking pulsed alarm: tone(9, 800) toggling every 500ms                        |
|    - Await flip: Physical 180 deg inversion silences buzzer and seamlessly starts next timer cycle|
+---------------------------------------------------------------------------------------------------+

# 5. Implementation Stage: Circuit Schematic & Pin Assignment

Below is the complete electrical interconnection schedule for constructing the prototype. Strict compliance with voltage rails (5V for displays, 3.3V for IMU) is mandatory to prevent component damage.

| Subsystem | Module Pin | Arduino Uno R4 Pin | Wire / Connection Type | Electrical Note |
| --- | --- | --- | --- | --- |
| Power Bus | VCC (Displays) | 5V | Red Jumper Wire | Direct 5V supply (high current) |
| Power Bus | GND (All) | GND | Black Jumper Wire | Common ground reference |
| Sensor Power | VCC (IMU) | 3.3V | Orange Jumper Wire | Regulated 3.3V logic supply |
| Visual SPI | DIN (Matrix #0) | Pin 11 (MOSI) | Yellow Jumper Wire | High-speed SPI Data |
| Visual SPI | CS (Matrix #0) | Pin 10 (SS) | Green Jumper Wire | Active-Low Chip Select |
| Visual SPI | CLK (Matrix #0) | Pin 13 (SCK) | Blue Jumper Wire | SPI Clock Line |
| Display Cascade | DOUT (#0) -> DIN (#1) | Inter-matrix | Direct Jumper | SPI Cascade Stage 1 |
| Display Cascade | DOUT (#1) -> DIN (#2) | Inter-matrix | Direct Jumper | SPI Cascade Stage 2 |
| I2C IMU | SDA (IMU) | SDA (or A4) | Purple Jumper Wire | I2C Serial Data (pullup) |
| I2C IMU | SCL (IMU) | SCL (or A5) | White Jumper Wire | I2C Serial Clock |
| Auditory | SIG (Buzzer) | Pin 9 (PWM) | Gray Jumper Wire | Square wave audio signal |
| Keypad Rows | Pins 1..4 | Pins 2, 3, 4, 5 | Ribbon Cable Left | Internal pullup scanned |
| Keypad Cols | Pins 5..8 | Pins 6, 7, 8, 12 | Ribbon Cable Right | Strobe outputs |

Design Stage Sign-off: Hardware, software, and circuit specifications are validated and ready for laboratory prototype assembly.
Box 34 Project Team - ELEC281